RangeSight

03 - Computer Vision & Impact Detection Specification

Deterministic pipeline, confidence model, failure modes, and validation hooks

See every shot.

Document status: Codex build reference. Treat requirements marked MUST as acceptance criteria; SHOULD as preferred behavior; MAY as future-safe extension points.

# 1. Objective

Detect a newly appearing, persistent visual mark on a stabilized paper target and express the result in target-space coordinates with an explicit confidence value. The detector must distinguish true new impacts from camera motion, target sway, lighting changes, existing holes, shadows, focus/exposure transitions, and compression/noise.

# 2. Design philosophy

Start with registration + change detection + geometry. Add ML only where measured failure modes justify it.

Do not treat audio as ground truth. Nearby lanes will generate impulses.

Do not infer invisible holes. If the visual evidence is insufficient, return no detection or low confidence.

Maintain a mask/history of previously confirmed impacts so an old hole is not rediscovered.

Allow target-specific tuning without forking the entire pipeline.

# 3. Target lock

Identify or let the user confirm the target quadrilateral/ROI.

Perspective-warp ROI into a normalized target plane when target geometry permits.

Estimate image quality: sharpness, brightness, clipped highlights, target size in pixels, registration feature count.

Reject lock if the target occupies too few pixels for the selected mode.

Capture a baseline frame stack and combine them (e.g., robust median/temporal denoise) rather than relying on one noisy frame.

# 4. Per-frame processing

| Stage | Output | Reject/quality signal |
| --- | --- | --- |
| ROI extraction | target crop | ROI outside frame |
| Registration | aligned current frame | registration error too high |
| Photometric normalization | brightness-normalized frame | large nonlocal lighting jump |
| Difference map | localized change intensity | global change ratio too high |
| Threshold/morphology | candidate blobs | blob too small/large/linear |
| Candidate features | shape, contrast, persistence, neighborhood | low discriminative signal |
| Temporal confirmation | persistent candidate | vanishes/moves across frames |
| History suppression | novel candidate only | overlaps known impact mask |

# 5. Candidate features

Area in normalized target pixels

Equivalent diameter

Circularity / eccentricity

Local contrast relative to pre-shot neighborhood

Dark/light polarity depending on target region

Edge density / raggedness

Distance from existing confirmed impact

Temporal persistence across N post-event frames

Position stability after registration

Whether candidate is localized versus part of a global exposure shift

# 6. Audio-assisted trigger

Audio impulse detection may narrow the search window but must never directly create a shot. In monitoring mode, maintain pre-event frames in a short ring buffer. When an impulse is detected, mark a candidate time window; after vibration settles, compare post-event frames against the most recent valid registered baseline. If no persistent visual change appears, emit no shot.

# 7. Confidence model

| Band | Behavior |
| --- | --- |
| 0.85-1.00 High | Auto-confirm impact and announce if announcements enabled. |
| 0.60-0.84 Medium | Show unobtrusive candidate for quick confirm/correct; no confident voice claim. |
| <0.60 Low | Do not create a shot; retain debug candidate only. |

Thresholds above are starting values, not contractual constants. Tune using the labeled validation set, prioritizing low false-positive rates.

# 8. Baseline update policy

After a confirmed impact, incorporate the new mark into the current reference state only after its position is stable.

Preserve a binary/soft mask of all confirmed impacts.

If registration quality collapses, pause detection and re-establish baseline rather than silently resetting.

If the target carrier moves significantly, require reacquisition or automatic re-lock followed by an explicit UI notice.

# 9. Known difficult conditions

| Condition | Risk | Required behavior |
| --- | --- | --- |
| .22/small holes at 25 yd | too few distinctive pixels | degrade confidence; never hallucinate |
| black-on-black hole region | low contrast | use temporal/edge evidence; allow miss |
| many overlapping holes | new mark merges with old | flag ambiguity; manual correction |
| target sway | large spatial change | registration + temporal stability check |
| light flicker/cloud-like exposure shift | global pixel change | photometric normalization/global-change rejection |
| tripod bumped/recoil vibration | misregistration | wait for stable frames; quality warning |
| neighboring gunfire | false audio trigger | visual confirmation required |
| splatter target | large visual burst | target profile-specific candidate sizing |

# 10. Debug instrumentation

Save optional debug snapshots: baseline, registered frame, diff map, candidate mask.

Log registration residual, ROI size, blur score, global change ratio, candidate feature vector, confidence, processing time.

Build a replay harness that can run recorded clips through the detector deterministically without a live camera.

Every false positive/negative discovered in field testing should become a reproducible regression fixture when licensing/privacy allows.

# 11. Acceptance test for the detector

A detector build is not accepted because it looks correct on a few clips. It is accepted only after running the labeled validation corpus and reporting precision, recall, false positives per 20-shot session, and breakdowns by distance, caliber, target region/contrast, device family, and target type.